Flash Controller Architecture Reference

Legacy SPI SRAM shadowing vs pure QSPI XIP vs dual-mode QSPI

This reference captures the current hardware-level test intent for the three flash-controller architectures. All designs are driven by the Cortex-M3 at SoC level, use 32-bit addressing, and are measured with a common 4-word flash payload.

# Common Test Baseline

| Item | Current specification |
| --- | --- |
| CPU | ARM Cortex-M3 integration model |
| Clock | 50 MHz HCLK, 20 ns period |
| Reset | Held for 20 HCLK cycles, then released on the aligned reset sequence |
| Addressing | 32-bit AHB addresses; QSPI flash accesses use 32-bit address fields |
| AHB data width | 32-bit CPU/system data |
| Common payload size | 4 x 32-bit words = 16 bytes of data |
| Completion marker | A final write of 0xAABBCCDD to 0x20008000 tells the testbench that the benchmark completed |

Common flash payload:

0x40000000 = 49024801

0x40000004 = e7fe6001

0x40000008 = 20008000

0x4000000c = aabbccdd

| Architecture | Fundamental idea | Current benchmark behavior | What timing means |
| --- | --- | --- | --- |
| Legacy SPI + SRAM shadowing | Copy flash code/data into SRAM first, then execute from SRAM. | Bootloader reads 4 words through SPI, writes them to SRAM, jumps to 0x20000001, and the SRAM payload writes the magic marker. | Initialization is shadow-copy time. Operation is SRAM payload execution time after shadowing is ready. |
| Pure QSPI XIP | No shadowing; CPU reads directly from memory-mapped QSPI flash. | Firmware performs four direct reads at 0x40000000..0x4000000c. It writes only the final marker to SRAM for testbench completion. | Initialization is first XIP read latency. Operation is remaining direct-read time. |
| Dual-mode QSPI | QSPI controller supports XIP reads plus program/erase capability. | For PPA read comparison, only the direct-read/XIP path is exercised. Write/erase path is intentionally not part of this benchmark. | Same timing definition as pure XIP, while area/power include the richer dual-mode hardware. |

# Architecture 1: Legacy SPI With SRAM Shadowing

The legacy architecture treats external flash as a boot source rather than the long-term execution target. The Cortex-M3 first runs a ROM bootloader. The bootloader reads words from flash through a memory-mapped SPI controller, stores them into SRAM, and then jumps into SRAM.

## Key Specification

| Item | Current specification |
| --- | --- |
| Controller bus | AHB-Lite slave mapped in the 0x40000000 region |
| Physical interface | Single-bit SPI: spi_cs_n, spi_clk, spi_mosi, spi_miso |
| Read command | 0x03 |
| Address phase | 32-bit flash address |
| Data phase | 32-bit serial receive |
| Destination | SRAM at 0x20000000 |
| Execution jump | 0x20000001, because Cortex-M3 requires bit 0 set for Thumb state |

## SPI Register Map

| Address | Register | Role |
| --- | --- | --- |
| 0x40000000 | SPI_CTRL_REG | bit[0] start, bit[1] busy |
| 0x40000004 | SPI_CMD_REG | Holds command byte, currently 0x03 for read |
| 0x40000008 | SPI_ADDR_REG | Holds 32-bit flash byte address |
| 0x4000000c | SPI_DATA_REG | Returns received 32-bit word |

## Legacy Test Flow

ROM bootloader starts after reset

for i = 0..3:

write SPI_CMD_REG  = 0x03

write SPI_ADDR_REG = flash_base + i*4

write SPI_CTRL_REG = 1

wait for busy to assert, then deassert

read SPI_DATA_REG

write SRAM[0x20000000 + i*4]

jump to 0x20000001

SRAM payload writes 0xaabbccdd to 0x20008000

## Legacy Timing Metrics

| Item | Current specification |
| --- | --- |
| Init time | Reset release to the last SRAM shadow write at 0x2000000c |
| Operation time | Last SRAM shadow write to the payload's magic write at 0x20008000 |
| Total time | Reset release to the magic write |

# Architecture 2: Pure QSPI XIP

Pure QSPI XIP removes the SRAM shadow step. The CPU accesses a memory-mapped flash window directly, and the AHB wrapper stalls the CPU until the QSPI read completes. The current benchmark does direct reads only; it does not copy payload words into SRAM.

## Key Specification

| Item | Current specification |
| --- | --- |
| Controller bus | AHB-Lite slave mapped in the 0x40000000 flash window |
| Physical interface | Quad SPI IO[3:0] with qspi_cs_n and qspi_clk |
| Read command | 0xEC on the first read |
| Address phase | 32-bit address transmitted as quad nibbles |
| Mode bits | 0xA5 to enter/maintain continuous XIP mode |
| Dummy cycles | 4 QSPI clocks before data |
| Data phase | 32-bit data received over four QSPI IO lines |
| Write/erase | Not supported in pure XIP architecture |

## Pure XIP FSM Flow

First read when xip_active == 0:

IDLE -> CMD(0xEC) -> ADDR(32-bit) -> MODE(0xA5) -> DUMMY -> DATA -> DONE

Later reads when xip_active == 1:

IDLE -> ADDR(32-bit) -> MODE(0xA5) -> DUMMY -> DATA -> DONE

## Pure XIP Test Flow

ROM firmware starts after reset

read 0x40000000

read 0x40000004

read 0x40000008

read 0x4000000c

if final word is 0xaabbccdd, write 0xaabbccdd to 0x20008000

## Pure XIP Timing Metrics

| Item | Current specification |
| --- | --- |
| Init time | Reset release to the first direct XIP read completion |
| Operation time | First XIP read completion to fourth XIP read completion |
| Total read time | Reset release to fourth XIP read completion |
| Marker latency | Fourth read completion to final marker write; useful to exclude from flash-read comparison |

# Architecture 3: Dual-Mode QSPI

Dual-mode QSPI keeps the XIP/direct-read path but adds flash program and erase capability. For the current PPA read comparison, the benchmark intentionally uses only the direct-read path so it can be compared fairly with pure XIP.

## Key Specification

| Item | Current specification |
| --- | --- |
| Controller bus | AHB-Lite slave mapped in the 0x40000000 flash window |
| Physical interface | Quad SPI IO[3:0] with qspi_cs_n and qspi_clk |
| Read command | 0xEC, same XIP read command as pure XIP |
| Write enable | 0x06 |
| Page program | 0x34 |
| Sector erase | 0x21 |
| Erase selector | HWDATA == 0xffffffff selects erase in the current RTL |
| XIP behavior | Reads can skip the command phase after xip_active is locked |

## Dual-Mode FSM Flow

Read path:

IDLE -> CMD(0xEC) -> ADDR -> MODE -> DUMMY -> DATA -> DONE

or, if xip_active == 1:

IDLE -> ADDR -> MODE -> DUMMY -> DATA -> DONE

Write/program path:

IDLE -> WREN(0x06) -> CMD(0x34) -> ADDR -> DATA/PROG -> DONE

Erase path:

IDLE -> WREN(0x06) -> CMD(0x21) -> ADDR -> PROG/DONE

## Dual-Mode Test Flow

ROM firmware starts after reset

read 0x40000000

read 0x40000004

read 0x40000008

read 0x4000000c

if final word is 0xaabbccdd, write 0xaabbccdd to 0x20008000

program/erase functions are not exercised in this read-comparison benchmark

## Dual-Mode Timing Metrics

| Item | Current specification |
| --- | --- |
| Init time | Reset release to the first direct QSPI read completion |
| Operation time | First QSPI read completion to fourth QSPI read completion |
| Total read time | Reset release to fourth read completion |
| Marker latency | Fourth read completion to final marker write |

# How To Interpret PPA Results

| Metric | Legacy SPI + SRAM Shadowing | Pure QSPI XIP | Dual-Mode QSPI |
| --- | --- | --- | --- |
| Performance | Pays upfront copy cost, then executes from SRAM. | Avoids copy cost but every flash read pays QSPI access latency. | Similar read path to pure XIP, plus possible overhead from richer control logic. |
| Power | SPI and SRAM active during shadowing; runtime can avoid flash for shadowed code. | QSPI flash path remains active for direct reads. | Read power similar in concept to pure XIP, but more logic can toggle. |
| Area | SPI controller plus SRAM dependency. | Read-only QSPI controller, usually simpler than dual mode. | Largest feature set: XIP read, write enable, program, erase, XIP lock/drop logic. |

# Important Fairness Notes

The architectures are intentionally not doing identical micro-operations. Legacy performs shadowing because that is its architecture. Pure XIP and dual-mode QSPI do not shadow into SRAM because their purpose is to replace the shadowing method with direct flash access.

The common payload and marker keep correctness and measurement boundaries consistent, while preserving the real architectural distinction: legacy measures flash-to-SRAM initialization plus SRAM execution, whereas QSPI designs measure direct flash read behavior.

For PPA reporting, use the testbench's printed init and operation times, then pair them with synthesis area and power estimates for each controller. When comparing pure XIP and dual mode, remember that the current test uses only the read path; the dual-mode area/power still includes unused write/erase capability.